#80s-cartoons Guide “How To Set Up A Wired Routers, Modems and Gateways BEFSR41 To Serve - Using Ports 59 and 6060 as examples.”  By KingBax

To allow Linksys Router firewall access to ports 59 and/or 6060, you will need to open ports in your router  by forwarding the ports you need to your computers assigned IP address..

*Note: This guide was made based on the Linksys Model BEFSR41 Router. Your settings may be a different. If so, contact linksys customer support on how to open ports on your router at Linksys Technical Support (800) 326-7114.

Step 1: You will need to set up your computer with a static IP..

Windows 2000/XP

- Click on the Start menu, and then click on the Run option.

- When the “Run” window appears, type control ncpa.cpl into the “Open” field.

- After the “Network and Dial up Connections” or “Network Connections” window opens, right click on the Local Area Connection or Wireless Network Connection which controls the connection to the router.

- After the “Local Area Connection Properties” or the “Windows Network Connection Properties” window appears, click on Internet Protocol (TCP/IP) then click on the Properties button.

Once the “Internet Protocol (TCP/IP) Properties” window opens, change the option from “Obtain an IP address automatically” to Use the following IP address. Set the options under “Use the following IP address” and “Use the following DNS server addresses” to the following:

IP: 192.168.1.20 (If there are multiple computers being setup with static IP's, then the next computer would be .25, and next would be .30, etc)

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

Preferred DNS server: 192.168.1.1

- Click OK then OK or Close.

Windows 98/ME

- Click on the Start menu, and then click on the Run option.

- When the “Run” window opens, input control netcpl.cpl into the “Open” field, then click on the OK button.

- When the “Network” window opens, locate the TCP/IP for the Wireless or Ethernet adapter in the PC, and click on it. Once it’s highlighted, click on the Properties button.

- 4.Change option from Obtain Automatically to Specify and input:

IP: 192.168.1.20 (If there are multiple computers being setup with static IP's, then the next computer would be .25, and next would be .30, etc)

Subnet Mask: 255.255.255.0

- Click on the Gateway tab, and input 192.168.1.1 into the “Default Gateway” field and click the Add button.

- Click on the DNS Configuration tab, then go to the “Host Name” field and input Computer, then go to the “DNS Server Search Order” field and input 192.168.1.1 then click the Add button.

- Click OK then OK once more, then windows will prompt for a restart.

Author: KBJLH

Copyright © 2005 Cisco Systems, Inc.

FOR PORT FORWARDING

Cick this link for steps on setting up your linksys router for Port Forwarding: http://makeashorterlink.com/?E23513B3A

Port Forwarding For Ports 59 and 6060 would look like this:

Customized Application	Ex.Port		TCP Protocol	UDP Protocol	IP Address	Enable

[Mirc]			[59 ] To [59 ]	        [x]		       [x]		192.168.1.[***]	[x]

Customized Application	Ex.Port		TCP Protocol	UDP Protocol	IP Address	Enable

[Mirc]			[6060] To [6060]        [x]		       [x]		192.168.1.[***]	[x]

*** denotes the IP number you entered for your static IP address in step 1 above.

Also you may need to check the linksys website for firmware updates that could help get you going. Even if you don’t think you need to update based on the “Whats New?” list, do so anyways because not necessarily all updates are listed.

Head to: http://www.linksys.com/download/ and follow the instructions to get your router’s download page.